# Mathematical Values and Thresholds for Drowsiness Detection

# 1. Eyes: Eye Aspect Ratio (EAR)

EAR calculates how open or closed the eye is by comparing vertical and horizontal distances between 6 key eye landmarks.

Mathematical Formula: EAR = (v1 + v2) / (2.0 * h)
• v1 = Vertical distance between the upper and lower eyelid (points 1 & 5).
• v2 = Vertical distance between the upper and lower eyelid (points 2 & 4).
• h = Horizontal distance across the eye (points 0 & 3).

Alert Threshold: An alert triggers if the average EAR for both eyes drops below 0.25 for 20 consecutive frames.

# 2. Mouth: Mouth Aspect Ratio (MAR)

MAR determines if the driver is yawning by measuring how wide the mouth is open.

Mathematical Formula: MAR = v / h
• v = Vertical distance between the Top Lip (landmark 13) and Bottom Lip (landmark 14).
• h = Horizontal distance between the Left Corner (landmark 78) and Right Corner (landmark 308).

Alert Threshold: An alert triggers if the MAR goes above 0.50 for 10 consecutive frames.

# 3. Head Nodding (Pitch)

Head pose is calculated mathematically using cv2.solvePnP which maps 6 2D facial landmarks to a 3D face model to extract pitch (looking up/down), yaw (turning left/right), and roll (tilting side-to-side).

## Static Nodding:

Thresholds: If the pitch stays within (-20 <= pitch <= -3) OR (5 <= pitch <= 20) continuously for 4.0 seconds.

## Dynamic Nodding (Sudden Drop):

The system records the pitch history of the last 15 frames (window_size). It finds the max_pitch and min_pitch in that 15-frame window.
Mathematical Formula: If (max_pitch - min_pitch > 20) AND (min_pitch < -15).
Meaning: If the head abruptly drops more than 20 degrees down, reaching an absolute angle lower than -15 degrees.